i6 CoreTX & Dual Cortex

Obstacle Avoidance System

User Handbook — Firmware v2.0.0

ESP32-S3 Dual-Cortex  ·  FS-i6 CoreTX  ·  Naza M V2  ·  iBUS + PPM

v2.0 changes from v1.9.2

iBUS input replaces SBUS  ·  RMT PPM output to Naza  ·  6-channel TX telemetry

ch10/ch11 now ESP GPIO outputs  ·  i6 firmware renamed CoreTX 1.0.0

# 1. What This System Is

A dual-processor obstacle avoidance retrofit for the Naza M V2 drone. Two ESP32-S3 boards split the work between perceiving the world and acting on it — sitting between the RC receiver and the flight controller without the pilot ever noticing it is there under normal conditions.

| Board | Role | Key inputs | Key outputs |
| --- | --- | --- | --- |
| Sensory Cortex | Perceives the world | 5× TF-Luna LiDAR, IMU, mag, baro, optical flow | SafetyPacket to Comm Cortex |
| Comm Cortex | Acts on sensor data | iBUS from FS-X6B, SafetyPacket from Sensory | PPM to Naza, GPIO aux, iBUS telemetry to TX |

## Core safety principle

The Comm Cortex only restrains pilot input — it never commands the aircraft independently. Avoidance feels like flying into thick air. You can always pull away, climb, or yaw. The Naza flies itself; the cortex only limits how far you can push toward an obstacle.

The system is not a substitute for situational awareness. It is a last line of defence, not permission to fly carelessly near obstacles.

# 2. What Changed in v2.0

## Signal path

The system now uses iBUS from the FS-X6B receiver (replacing SBUS) and outputs PPM to the Naza (via CD4053 bypass switch). The 74HCT04 gates 1 and 2, the SBUS-to-PWM breakout board, and the voltage divider on GPIO20 are all removed. The signal path is entirely 3.3V with no level shifting needed.

| What was removed | What replaced it |
| --- | --- |
| SBUS input (100kbaud 8E2, inverted) | iBUS input (115200 8N1, direct 3.3V, GPIO4) |
| SBUS output (74HCT04 gate 2) | RMT PPM output (hardware peripheral, GPIO6) |
| 74HCT04 gates 1+2 | Not needed — PPM is 3.3V non-inverted |
| SBUS-to-PWM breakout board | Not needed — ch10/ch11 now ESP GPIO outputs |
| 4.7kΩ/10kΩ voltage divider (GPIO20) | Not needed — iBUS is 3.3V direct |
| ch10 floodlight → RC switch module | ch10 → ESP GPIO1 → FET gate directly |
| ch11 VTX → PWM breakout | ch11 → ESP GPIO2 → FET gate directly |

## iBUS input

The FS-X6B outputs 14 channels on iBUS at ~142Hz. Values are 1000–2000µs throughout the firmware — no SBUS conversion. All avoidance arithmetic works natively in PPM microseconds.

## RMT PPM output

The ESP32-S3 RMT hardware peripheral generates the 8-channel PPM pulse train to the Naza completely independently of FreeRTOS scheduling. An ISR callback reloads channel values at 50Hz. Jitter is under 1µs. This output feeds the CD4053 bypass switch — raw PPM from the receiver on one side, ESP-modified PPM on the other.

## iBUS telemetry (new in v2.0)

The Comm Cortex now responds to iBUS telemetry polls from the i6 CoreTX transmitter. Six sensor values are sent back to the TX and displayed on its telemetry screen in real time.

# 3. Hardware

## Parts list (changes from v1.9.2)

| Item | Change | Notes |
| --- | --- | --- |
| 74HCT04 hex inverter | Gates 1+2 unused — tie inputs to GND | Gates 3–6 still drive nav LEDs as before. Or replace with 2N2222 transistors and remove the chip entirely. |
| SBUS-to-PWM breakout board | Remove | No longer needed |
| 4.7kΩ + 10kΩ resistors (SBUS divider) | Remove | GPIO20 now used for telemetry RX |
| 1kΩ resistor | Add | Series on GPIO21 TX to iBUS telemetry wire |

## GPIO map — Comm Cortex (ESP32-S3 Super Mini)

| GPIO | Function | Connected to |
| --- | --- | --- |
| 4 | iBUS RX (UART0) | FS-X6B iBUS pad — direct, 3.3V, no inversion |
| 5 | Watchdog pulse 50Hz | 47kΩ + 1µF RC filter → CD4053 Switch A SELECT |
| 6 | RMT PPM output | CD4053 Switch B IN-B (3.3V) |
| 1 | Floodlight GPIO (ch10) | 2N2222/MOSFET gate → floodlight |
| 2 | VTX GPIO (ch11) | 2N2222/MOSFET gate → VTX power |
| 7 | Nav LED Front-Left | Via 2N2222/HCT04 inverter → green LED |
| 8 | Nav LED Front-Right | Via 2N2222/HCT04 inverter → red LED |
| 9 | Nav LED Rear-Left | Via 2N2222/HCT04 inverter → green LED |
| 10 | Nav LED Rear-Right | Via 2N2222/HCT04 inverter → red LED |
| 16 | Sensory Cortex UART RX | Sensory Cortex GPIO43 |
| 17 | Sensory Cortex UART TX | Sensory Cortex GPIO2 |
| 20 | iBUS telemetry RX (UART2) | FS-X6B iBUS sensor port — direct |
| 21 | iBUS telemetry TX (UART2) | 1kΩ series resistor → FS-X6B iBUS sensor port |
| 48 | WS2812 status LED | Onboard |

## CD4053 bypass wiring (v2.0)

All signals are 3.3V — no level shifting anywhere in the PPM path.

Switch B (PPM selector):

IN-A  ← Raw PPM from FS-X6B PPM output (3.3V)

IN-B  ← ESP PPM from GPIO6 (3.3V)

OUT   → Naza M V2 PPM port

SELECT ← Switch A OUT

Switch A (watchdog gate):

SELECT ← GPIO5 RC filter (47kΩ + 1µF)

+ Pololu RC switch output (ch8 from upstream PPM decoder)

OUT   → Switch B SELECT

INH   → GND,  VCC → 3.3V,  GND → GND

## iBUS telemetry wiring

Both GPIO20 (RX) and GPIO21 (TX) connect to the FS-X6B iBUS sensor port. The 1kΩ series resistor on GPIO21 TX prevents bus contention when the ESP and receiver briefly overlap on the line.

GPIO21 TX → 1kΩ → iBUS sensor port wire

GPIO20 RX ←────── iBUS sensor port wire (direct)

FS-X6B iBUS sensor port GND → shared GND

## Channel map

| Ch | idx | Function | Handler | Notes |
| --- | --- | --- | --- | --- |
| 1 | 0 | Roll | Naza PPM | Avoidance-modified |
| 2 | 1 | Pitch | Naza PPM | Avoidance-modified |
| 3 | 2 | Throttle | Naza PPM | Floor/ceiling-modified |
| 4 | 3 | Yaw | Naza PPM | Always pass-through |
| 5 | 4 | Flight Mode | Naza PPM | Pass-through. RTH detection. |
| 6 | 5 | IOC | Naza PPM | Pass-through |
| 7 | 6 | Gimbal Tilt | Naza PPM | Pass-through |
| 8 | 7 | CD4053 bypass | Upstream HW | Pololu RC switch upstream. ESP ignores. |
| 9 | 8 | Proximity VRB | Comm Cortex | Zone scale in flight; floor set in landing |
| 10 | 9 | Floodlight | GPIO1 out | HIGH above threshold |
| 11 | 10 | VTX on/off | GPIO2 out | HIGH above threshold |
| 12 | 11 | Nav lights | GPIO7–10 | Corner LED state machine |
| 13 | 12 | Landing mode | Logic flag | Suspends lateral brake |
| 14 | 13 | Ceiling switch | Logic flag | Captures/releases altitude ceiling |

# 4. Firmware Installation

## i6 CoreTX transmitter firmware

Flash CoreTX_1.0.0.bin to your FS-i6 transmitter. This is a fully pre-patched binary — flash it directly, the patcher is only needed if you ever need to rebuild from scratch.

# Power off TX

# Hold OK and power on — update screen appears

# Connect USB

flysky-updater-win64.exe CoreTX_1.0.0.bin   (Windows)

./flysky-updater-linux64  CoreTX_1.0.0.bin   (Linux)

CoreTX 1.0.0 includes all i6 MAX 1.0.0 features (16-channel SBUS, PPM 4/5, failsafe ch4–9) plus renamed telemetry sensor labels to match avoidance data.

## Arduino IDE settings

### Comm Cortex (ESP32-S3 Super Mini)

| Setting | Value |
| --- | --- |
| Board | ESP32S3 Dev Module |
| Flash Size | 8MB |
| PSRAM | Disabled |
| Partition Scheme | Huge APP (3MB No OTA) |
| USB CDC On Boot | Enabled |

### Sensory Cortex (ESP32-S3 N16R8)

| Setting | Value |
| --- | --- |
| Board | ESP32S3 Dev Module |
| Flash Size | 16MB |
| PSRAM | `OPI PSRAM` — critical, firmware won't run without this |
| Partition Scheme | Huge APP (3MB No OTA) |
| USB CDC On Boot | Enabled |

## Libraries

| Library | Used by | Install via |
| --- | --- | --- |
| ICM42688 by Bolder Flight Systems | Sensory Cortex | Library Manager |
| Adafruit BMP280 | Sensory Cortex | Library Manager |
| Bitcraze PMW3901 | Sensory Cortex | Library Manager |
| ArduinoBLE | Both Cortices | Library Manager |
| FastLED | Comm Cortex | Library Manager |

# 5. iBUS Telemetry — TX Display

The Comm Cortex sends six live sensor values back to the i6 CoreTX via iBUS telemetry. View them under System → i-BUS Setup → Sensors list on the transmitter.

| TX label | Data | Range / units | Notes |
| --- | --- | --- | --- |
| Zone | Avoidance zone status | 0 = clear, 1 = influence, 2 = brake, 3 = sys fault | Most important readout. 3 means sensor cortex dead or faulted. |
| Obs. | Closest obstacle | 0–999 cm (999 = clear) | Estimated from TTE × closing velocity. Treat as advisory. |
| Head | Threat bearing | 0–359 degrees (0 = no threat) | Direction of most urgent obstacle from nose. 0=front, 90=right, 180=back, 270=left. |
| RoC | Vertical velocity | cm/s signed (+ = climb, − = descent) | From Sensory Cortex AHRS. Useful for monitoring floor/ceiling response. |
| Flor | Altitude floor | 0–800 cm (0 = floor off) | Height of the active minimum altitude floor. |
| Alt. | Altitude AGL | 0–800 cm | Raw TF-Luna Down reading passed from Sensory Cortex. Accurate below 8m. |

IntV and ExtV on the TX remain the receiver's real battery voltage telemetry. These are provided by the FS-X6B directly and are not affected by the CoreTX firmware.

# 6. First-Time Setup

## Step 1 — TF-Luna address assignment

All five TF-Lunas ship at address 0x10. Assign one at a time before fitting — connect each via USB-TTL and use the TF-Luna config tool:

| Sensor position | I2C address |
| --- | --- |
| Front | 0x10 (unchanged from factory) |
| Left | 0x11 |
| Right | 0x12 |
| Back | 0x13 |
| Down | 0x14 |

## Step 2 — Comm Cortex standalone test

Set SENSORY_CORTEX_PRESENT 0 in comm_cortex_v2_0_0.ino, flash, then:

- Connect FS-X6B iBUS output to GPIO4

- Connect FS-X6B PPM output to CD4053 IN-A and Naza PPM port via CD4053 OUT

- Verify Naza responds to sticks normally

- Flip ch8 — verify bypass engages (WS2812 goes blue)

- Test ch10 floodlight GPIO1 and ch11 VTX GPIO2 switch with TX

- Test ch12 nav lights — corners should blink (Mode 1, single blink)

- When confirmed working, set SENSORY_CORTEX_PRESENT 1 and reflash

## Step 3 — Flash Sensory Cortex

PSRAM must be set to OPI PSRAM in Arduino IDE before flashing. The firmware will not boot without it.

Flash sensory_cortex_v1_9_2.ino. Open Serial Monitor at 115200 baud. All sensors should report [OK].

## Step 4 — Full system power-up check

| Indicator | Expected state |
| --- | --- |
| Comm Cortex WS2812 | Solid dim green |
| BLE SensoryCortex → Status | OK |
| BLE CommCortex → Status | OK |
| BLE CommCortex → Watchdog | OK 50Hz |
| TX telemetry → Zone | 0 (clear) |
| TX telemetry → Alt. | Near 0 cm on bench |

## Step 5 — Compass calibration (BLE)

- Power complete drone (motors disarmed, ESCs powered)

- Connect to SensoryCortex via nRF Connect or LightBlue

- Write go to the Compass cal characteristic

- Slowly rotate the drone through every orientation for 30 seconds

- Note the X/Y/Z offsets returned and write them into CFG_HMC_OFF_X/Y/Z for permanence

## Step 6 — Optical flow calibration

- Set OF weight to 0.0 via BLE: write 0.0 to OF weight

- Fly a measured straight line (100cm recommended)

- Land immediately, write that distance in cm to Flow cal (e.g. 100)

- Write the returned scale into CFG_OF_SCALE for permanence

- Restore OF weight to 0.8

# 7. Flying — What to Expect

## Nav lights (ch12 on)

| Mode | Condition | Pattern |
| --- | --- | --- |
| Off | Ch12 low | All four corners off |
| Mode 1 | No Sensory Cortex data | Single blink: FR then FL, 2s cycle |
| Mode 2 | All clear | Double blink: FR×2 then FL×2, 1.8s cycle |
| Mode 3 | Influence zone active | Threatened corners fast-blink 80ms |
| Mode 4 | Brake zone active | Threatened corners solid, others off |

## Approaching an obstacle

Influence zone: the corner LED nearest the obstacle blinks fast. Your control in that direction gets progressively heavier. The TX Zone display reads 1.

Brake zone: the corner LED goes solid. The drone cannot move toward the obstacle. You can always pull away, climb, or yaw. Zone reads 2.

## Proximity dial (ch9)

In flight mode: scales all avoidance distances from 0.3× (full down, tight indoor flying) to 2.0× (full up, fast outdoor flying). In landing mode: sets the minimum altitude floor.

## Landing mode (ch13)

Switches off lateral braking so you can descend between obstacles without the system fighting you. The TX Zone display shows landing mode via the Comm Cortex WS2812 LED going purple. Altitude floor behaviour changes — see altitude floor section.

## Ceiling switch (ch14)

Flick ON wherever you are — captures current altitude as the ceiling. The drone cannot climb above it. Flick OFF to release. Capture only happens when flying roughly level and below 8m.

## Corner LED direction guide

| LED corners lit | Obstacle direction |
| --- | --- |
| Both front (FL + FR) | Ahead |
| Front-right only (FR) | Ahead-right |
| Both right (FR + RR) | To the right |
| Rear-right only (RR) | Behind-right |
| Both rear (RL + RR) | Behind |
| Rear-left only (RL) | Behind-left |
| Both left (FL + RL) | To the left |
| Front-left only (FL) | Ahead-left |

# 8. Altitude Floor and Ceiling

## Setting the floor

- Take off in landing mode (ch13 on). Dial at minimum = floor off.

- Climb to your desired minimum height.

- Wind the dial up — the floor locks in beneath you.

- Switch to flight mode (ch13 off). The floor is remembered for the whole flight.

In flight mode the dial returns to controlling avoidance zone scale. The floor value is held. Watch Flor on the TX telemetry to confirm the set height in cm.

## Active floor (arm via BLE first)

g_activeFloorArmed defaults OFF. The active floor adds throttle to arrest descent — bench-test props-off before any flight.

Send afloor arm via BLE to enable. The active floor adds throttle proportional to descent rate when you approach the floor — it can actually arrest momentum, unlike the passive clamp which only blocks further throttle-down input.

## Landing

- Switch to landing mode (ch13 on).

- Wind dial to minimum — floor drops away completely.

- Descend to ground, disarm.

## RTH protection

When the Naza goes into return-to-home (whether pilot-triggered via ch7 or on signal loss), the floor, ceiling, and all avoidance step completely aside. The Naza is given a clean PPM path to fly home and land. The floor can never trap a returning drone in a hover.

TX telemetry Zone reads 0 (clear) during RTH — this is intentional, not a fault.

# 9. BLE Reference

## SensoryCortex (read-only, notify every 1s)

| Characteristic | Shows |
| --- | --- |
| Status | OK or FAULT: front left compass baro |
| Altitude | e.g. 1.42m |
| Heading | e.g. 284 deg |
| Map | e.g. 247 / 5000 points |

## SensoryCortex (write to calibrate/tune)

| Characteristic | Write | Effect |
| --- | --- | --- |
| Compass cal | go | Starts 30s compass calibration |
| Flow cal | distance in cm | Sets optical flow scale from flown distance |
| Map TTL | seconds 5–600 | How long mapped obstacles are remembered |
| Baro weight | 0.0–1.0 | Barometer trust in altitude fusion |
| OF weight | 0.0–1.0 | Optical flow trust in velocity fusion |
| Map clear | clear | Wipes the obstacle map |

## CommCortex (read-only, notify every 1s)

| Characteristic | Shows |
| --- | --- |
| Status | OK / SENSOR DEAD / RC LOST |
| Zone | CLEAR / INFLUENCE FRONT / BRAKE FRONT-RIGHT etc. |
| Watchdog | OK 50Hz / STOPPED |

## CommCortex avoidance commands

| Write | Effect |
| --- | --- |
| on` / `off | Master avoidance switch |
| horiz on` / `horiz off | Front/back/left/right avoidance only |
| alt on` / `alt off | Floor + ceiling corridor only |
| map on` / `map off | Map-based secondary nudge |
| afloor arm` / `disarm | Active floor throttle assist (default off — bench-test first) |
| afloor cap 0.0–0.6 | Active floor authority cap (fraction of stick range) |
| afloor band 0.2–6.0 | Active floor engage band in metres |
| afloor kp 0.0–5.0 | Active floor P gain |
| afloor kd 0.0–5.0 | Active floor D gain |
| afloor bank 5–30 | Max bank angle (deg) to stay active |
| afloor status | Report current active floor settings |
| ahoriz arm` / `disarm | Active horizontal brake (default off — bench-test first) |
| ahoriz cap 0.0–0.6 | Active horiz authority cap |
| ahoriz kp 0.0–2.0 | Active horiz P gain |
| ahoriz kd 0.0–2.0 | Active horiz D gain |
| ahoriz vref 50–1000 | Closing speed (cm/s) giving full-cap push |
| ahoriz engage 0–200 | Closing speed deadband (cm/s) |
| ahoriz status | Report current active horiz settings |

## CommCortex other commands

| Characteristic | Write | Effect |
| --- | --- | --- |
| Sensory | on` / `off | Enable/disable Sensory Cortex input |
| Failsafe RTH | PPM value 1000–2000 | Ch5 value on signal loss (default 1000 = RTH) |
| Timeouts | sensor:50 ibus:100 | Watchdog timeouts in ms |

# 10. Safety Systems

Five independent layers, each weaker than the one before, each acting as a fallback:

| Layer | Trigger | Response | Mechanism |
| --- | --- | --- | --- |
| 1 | Sensor hardware fault | Fault bits set in SafetyPacket — sensor excluded | Sensory Cortex internal self-test |
| 2 | SafetyPacket absent >50ms | Avoidance disabled — pure PPM relay | Comm Cortex software watchdog |
| 3 | iBUS absent >100ms (RC lost) | Failsafe PPM frame sent to Naza (ch5=1000=RTH). Flood+VTX GPIOs forced LOW. | Comm Cortex software watchdog |
| 4 | Comm Cortex crash | GPIO5 watchdog pulse stops → RC filter discharges → CD4053 routes raw PPM to Naza | Pure hardware — no firmware involved |
| 5 | Pilot flips ch8 | Pololu RC switch blocks watchdog → same result as layer 4 | Hardware — ESP-independent, works even if both cortices are dead |

Layers 4 and 5 both ultimately work through the same CD4053 hardware path. They are independent of each other and independent of all ESP32 firmware.

# 11. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| WS2812 slow green blink | Sensory Cortex not communicating | Check UART wiring GPIO16/17 ↔ GPIO43/2, common GND |
| TX Zone reads 3 (fault) | Sensory Cortex dead or not connected | Check UART link, check PSRAM=OPI in IDE, reflash Sensory |
| TX telemetry shows nothing | iBUS telemetry not wired | Check GPIO20/21 to X6B iBUS sensor port, check 1kΩ on GPIO21 |
| Naza not responding to sticks | PPM path issue | Check CD4053 wiring, check ch8 bypass position, check GPIO6 RMT output |
| No avoidance at all | Avoidance disabled or sensor absent | BLE CommCortex: write `on` to Avoidance, `on` to Sensory |
| Heading drifts / map rotates | Compass not calibrated | Run compass calibration via BLE |
| Drone drifts in hover | Optical flow not calibrated | Run flow calibration via BLE |
| BLE CommCortex Watchdog: STOPPED | GPIO5 not pulsing | Check 47kΩ+1µF RC filter, check Comm Cortex is running |
| Floor not holding | Active floor not armed | BLE CommCortex: `afloor arm` (bench-test props-off first) |
| Firmware won't boot (Sensory) | PSRAM not enabled | Arduino IDE: PSRAM → OPI PSRAM (critical) |
| iBUS decode not working | Wrong UART settings | Must be 115200 8N1 non-inverted. GPIO4 direct from X6B. |

# 12. Saved Settings

The Sensory Cortex saves calibration and tuning values to ESP32 NVS flash automatically. You only calibrate once per build. The following are saved and reloaded at every boot:

- HMC5883L hard-iron offsets (compass calibration)

- Optical flow scale factor

- Barometer weight, flow weight, map point TTL

Not saved — by design: avoidance enable, active floor arm state, altitude floor and ceiling. These always boot to their safe defaults regardless of last session state.

To wipe all saved values and return to compiled defaults, send reset to the Map-clear characteristic over BLE. Defaults take effect on next boot.

# 13. Future Expansion

| Addition | Interface | Status |
| --- | --- | --- |
| Visual Cortex (ESP32-CAM) | UART → Sensory GPIO3 | Reserved — GPIO3 allocated |
| Upward/extra LiDAR | I2C addresses 0x15, 0x16 | Bus has room |
| iBUS telemetry expansion | Serial2 GPIO20/21 — already wired | 6 sensors active, 10 more slots available |
| SD card logging | Sensory SPI bus spare | Available |
| X6B receiver reflash | Open receiver, flash via pads | Enables 18-ch iBUS — adds ch15/ch16 from trainer port encoder |

Ch15 and ch16 from the i6 CoreTX trainer port PPM encoder are transmitted over the RF link but only accessible via iBUS if the FS-X6B receiver is reflashed with community firmware (Povl Pedersen / extended iBUS). The CoreTX TX firmware already supports them. No TX-side changes needed.